# SKC PC Based Single Kiln Controller Software

And Peripheral Manual

## Hardware Setup

Install power connectors

Power leads come without plug or socket installed as many different types are used with kilns.  Add the correct plug and socket on the power cables.  Use a qualified electrician to insure safety.

GREEN = ground

BLACK = switched side of AC power line

WHITE= unswitched side of AC power line

### Install serial communications cable

Connect the serial communications cable from the open serial port you intend to use to the serial port as labeled on the kiln controller peripheral.  The serial port must be serial port either 1, 2 or 3.

Install the thermocouple probe and cable assembly

Warning:  The sensor must be placed in the kiln and connected before normal kiln

operation.  Failure to do so will cause the kiln to heat to its maximum

full on temperature.    Your kiln should ALWAYS be operated where

this full on condition would not create a safety hazard as human operator

error is impossible to prevent over time.

Place the thermocouple sensor in the kiln and verify electrical connection to its

cable.   Route the cable back to the kiln controller peripheral in such a way as to

prevent it being disturbed by any means.  If this connection is broken it will lead

to a full on heat condition and most likely ruin any work in progress in the kiln.

### Connect low power AC to the Kiln controller peripheral

Plug in the 120 VAC cord to a source.   Its recommended that for long

interval runs such as glass annealing that you use a “UPS”.  ( uninterruptable

power supply )   This allows the main user console control computer not to

lose its “mind” in the case of a power outage.

### Plug in High Power AC lines

The kiln controller peripheral cover should always be in place and screwed on

for this step.

Setup your computer hardware per what is appropriate for it

System requirements ( minimum )

- windows 95, 98

- pentium 100 Mhz or better

- for email capability MS Outlook express and an internet connection

- monitor resolution of 800 by 600 or greater

- system should be chosen for stability….realize that there are very stable

systems that will work very well for this application. (  glass annealing runs of 10 weeks without a single restart )

- sufficient hard drive space to allow you to log temperatures

- CD rom or floppy installed

- Dedicated system.  This system should not be used for other applications

concurrent with kiln operation if possible.  Most windows based software have memory leaks that build up over time forcing an eventual reboot.  Use and old stable cheap system that can be dedicated.  Future enhancements will include ability to view the kiln system parameters remotely such as over the internet or a network connection.

## Software installation

Place CD or floppy #1 in appropriate drive then

click on         “start”

click on          “run”

type                a: setup.exe

click on         “ok” button

The drive should then activate and initiate the installation program.

Use the default directory specified for proper operation.   Specifying

another directory will result in the software not being able to recall its

vital parameters at startup.

### Setting the Serial Port Number for Operation

Once the hardware is set up and the software installed the serial comm

port setting should be set up for proper operation.   This can be done by

going in the [ Setup -> CommPortNumber ] and sequencing through

the settings 1,2,3 till proper operation is observed.

Watch the User Prompt Window while you click on the “RUN

PROFILE” button turning on the run.  Each time it is cycled on it sends

a message to the peripheral controller immediately.

successful communication:      No error message will come up and the

oven temperature will be displayed.  Once

determined to be successful exit the program

and restart it to store the setting for future

use.  This should not have to be set again

unless your computer  hard drive is faulty.

unsuccessful communication:  Communications error message shows in

prompt window,  change CommPortNumber

setting and try again till successful.

Software Operation

Double click on the SKC icon to start the Single Kiln Controller software.

#### The software at installation is set for a warm bootup. This causes the display fields to be

filled with previously stored values.  Explore the menu items to see what is under each item.

#### User Prompt Boxes

When using the Temperature Trajectory or Profile Editor refer to the user prompt

box for messages regarding the operation of the software.

#### Opening a profile for run in the temperature trajectory window

#### The first menu heading is the “File” menu.   To load a temperature profile to be run

use  [ File -> OpenProfile] .   This brings up a file dialog box in the “Profiles” directory.

This is where you specify which profile to load and be displayed in the profile display

An example would be the Glass_Window_1.txt profile shown previously that loaded with

the warm boot up.

All profiles must be in a strict format to successfully be allowed to load.  The best way

to create a profile is using the profile editor which will be treated as a topic later when the

profile editor is discussed.    The reason for this is to prevent errors in loading profile

values during the profile run leading to your job being hung up.

The  [ File -> Print ]   prints the profile out for keeping of notes.

### Specifying a LogFile name for temperature logging

#### By clicking on [ LogFile -> Specify LogFile ]  you bring up the logging file dialog

box.  This is where you specify the name of the file used to log the temperature

run.  You can either double click on and existing logfile which will cause the

subsequently logged data to be appended to the existing file or type in a new

name in the file name blank which will start with a blank file with the first entry.

Use the default directory as it comes up or you will encounter load errors on

warm bootup.  After successful specification of the logfile  the logfile display will display

the name of the logfile.   If its necessary to clean the “LogFile”directory use Windows

explorer to move old run logfiles to an archival directory and retain the default logging

directory for use with the operation of the software. The each line in the log record

records takes the following form:

time    desired temp   measured temp

#### You can view the temperature log file contents by using any ordinary text editor

such as Wordpad .

#### Time as the software sees it only elapses while the “RUN” button is activated by

clicking on it.  You can tell its activated by the color.  When active the “RUN” button

is bright red.  If you suspend the run by clicking on the “RUN” button again to turn it

back to grey time will not be incremented and temperature not logged in this state.

### If you have a profile that you use repeatedly you can use the

### [ LogFile -> Reset LogFile and Time ] command from the menu.  This option

resets the elapsed time to zero and automatically generates a new log file name by

combining the profile name with current date and time to generate a unique name for the

log.

### Software Setup Menu

#### Setup has 2 options ->   RecallLastStateAtStart

- CommPortNumber

If the RecallLastStateAtStart is checked then the software recalls all the control variables

and software state when the software was last updated which occurs when you update an

essential variable or exit the program using the “EXIT” menu option.  The recalled

items are :

ProfileName

Profile ( corresponding to the associated name )

LogFile Name

Elapsed time ( from the last record entry in the LogFile )

The CommPortNumber has a submenu allowing you to specify the comm port number

1, 2 or 3.   The comm port number is recalled at start up so you do not have to reset it

once you determine the correct setting.

### General Email feature operation notes

#### The operation of the email notification feature is dependent on your computer

having MS Outlook running.  MS Outlook has to be set correctly to allow the SKC

application to send and email without you being there.  The default setting in Outlook is

to warn you that another application is trying to send an email and wait for you to ok the

transmission.   Since you are not going to be in proximity of the computer you must

turn this warning feature off.  This is done in the Outlook software by clicking on

Tools -> Options -> Security TAB  and clearing the check mark next to the

Warn me when other applications try to send mail as me   entry.

In addition to correctly setting up the above you must establish a connection to the

internet when a message is que’d for transmission.  If you have an always on

cable connection no additional  effort is required.  If you use dial up you will need to

set your Outlook software to periodically dial up and connect to transmit any que’d up

notifications there maybe in the “Outbox”.   At present the software transmits notification

of out of limits condition only.  This occurs when the actual measured temperature

deviates more than the Out of Range Error Limit  from the setpoint temperature.

The Out of Range Error Limit is set on the temperature trajectory form using the

increment and decrement buttons.  This limit applies both on the high and low side of the

setpoint so if the actual temp is higher or lower than the setpoint by the Out of Range

value a notification is sent in an email consisting of the setpoint and actual temperatures.

### Email Menu

#### Setting the checkmark next to [Email -> EditNotificationList] enables you to

#### click your cursor on the Email Notification Addresses    text box and edit the email

destination where any email notifications are to go.  The notifications can go to multiple

destinations as long as you correctly separate the email addresses correctly with a

comma.  This can include a text message sent via email to a cellular phone making it a

completely portable function.  There is no parsing done on the entry you put in this box

so recheck your entry when you think you are finished to make sure it appears correct.

When you are done clear the checkmark next to EditNotificationList to lock the entry

Next test your email notification feature by using [Email -> TestNotification ] .  This

will transmit a test message to the destination(s).   Verify receipt of the test notification at

the intended email addresses before proceeding.

Once verified setting the checkmark next to the [Email -> EnableNotification ]

enables the software to automatically notify you of out of range conditions in your kiln.

### Edit / Create Profile

#### Clicking on this menu item opens up the profile editor.  Use the profile editor

to generate a profile with the precise format required to correctly operate in the

temperature trajectory form.  Strict formating is required to prevent erroneous

operation.  More details on profile creation and Profile Editor features in the

appropriate section.

### Lock Controls

Setting the checkmark next to the [ Lock Controls -> LockControls? ]  locks the

controls so inadvertent mouse clicks do not change the state of the “RUN PROFILE”

control button.  In a protracted run such as used in glass annealing this is relevent.

### Exit

The [ Exit -> ExitProgram ]   item causes the computer to exit the program.  This

is the only way to do so in a graceful fashion.  X’ing out of operation has been disabled

to prevent inadvertent exits.

### About

[ About -> Version ]   causes the software version number to be displayed in the

user prompt display.

### Control Variable Displays on the Temperature Trajectory Interface

The current state of the run is displayed via the run parameter displays.

Segment displays the current segment from the profile being executed.

Setpoint displays the desired temperature in Fahrenheit as calculated from the profile and

the current time.

Oven Temp  displays the measure kiln temperature

Elapsed Time displays the cumulative time that has elapsed while the “RUN PROFILE”

button is in the on state.  On / Off state can be discerned by the color of

the  “RUN PROFILE” button which is red while on.

Note that the Elapsed Time can be manually set.  The set action can only

occur while the run is suspended by clicking on the “RUN PROFILE”

control button.  Place the mouse cursor over the Elapsed Time display and

click on it.  Use the number and backspace keys to set the desired time

manually and hit enter when set correctly.  You may restart the run by

clicking on “RUN PROFILE” again at this point.

### Profile Editor Overview

#### The profile editor is an essential tool for creating profiles that are formatted correctly

to insure proper operation with the trajectory form.  Although they can be viewed with

an ordinary text editor such as wordpad you cannot create them with wordpad. This is

because there are several built in safeguards against it.  This is to insure proper operation

when running the profile.  Using the profile editor  you can open up an existing

profile for edit or create a new one.

Profile segments are created by clicking on the appropriate field of the profile

grid and typing in the entry.  The entries are automatically parsed to prevent illegal

entries from occurring and thereby preventing a run from hanging up due to the erroneous

entry.

As you fill in the fields of the segment the software calculates

any entries it can as it becomes possible.  For instance if you enter

start temp=100, end temp= 800, and ramp rate= 100 then it can calculate

duration = (800-100)/100=7 hours ( given units selected are hours) …in

addition it updates its own and all subsequent segments start time.

This results in easily modified and updated profiles.

Note that when you reach the last entry the “e” in the action column signifies

the end  of the profile.

Profile parsing includes:

- verifying there are no skipped profile segments

- verifies the specified temperature is within the sensor temp limits

[ File -> Open ]  Clicking through this sequence allows you to open up an existing

profile to use it as a base for a new profile with modifications.

[ File -> SaveAs ]  This sequence allows you to save a profile you have edited.  If you

have opened an old file to edit into a new one make sure to specify a new name to avoid

overwriting your old file.

[ File -> New ]  This sequence clears the profile for editing from scratch.

##### [ Exit -> Exit Editor] This sequence closes the profile editor.  Nothing is automatically

saved but you are prompted if you have not saved off your modified profile. Make sure

to use the [ File -> SaveAs ]  to save the modified profile.